School of Engineering Institute of Technology Nirma University ,Ahmedabad

CA INNOVATIVE ASSIGNMENT

TOPIC: RESTORING DIVISION ALGORITHM

SUBMITTED BY:24BCE122 – NEEL SHAH

24BCE133 – VEDANG PATEL

1. Introduction to Restoring Division

The Restoring Division Algorithm is a fundamental binary division technique used in computer arithmetic. It mimics long division but operates on binary numbers, restoring the partial remainder when subtraction results in a negative value — hence the name 'Restoring'.

## 1.1 Key Concepts

- Dividend (Q): The number to be divided

- Divisor (M): The number dividing into the dividend

- Accumulator (A): Partial remainder register, initially set to 0

- Quotient Register (Q): Stores the dividend bits; quotient bits set here

- n: Number of bits (determines number of steps)

## 1.2 Algorithm Steps

- Initialize: Set A = 0 (n-bit accumulator), load Dividend into Q register

- Left Shift: Shift the combined [A, Q] register left by 1 bit

- Subtract: Compute A = A - M (subtract divisor from accumulator)

- Check Sign of A: If A >= 0 (MSB=0): Set Q[0] = 1 (quotient bit = 1), keep A as is (KEEP)

- If A < 0 (MSB=1): Set Q[0] = 0 (quotient bit = 0), RESTORE A = A + M (RESTORE)

- Repeat steps 2–4 for n iterations (n = number of bits in dividend)

- Result: Q holds the Quotient; A holds the Remainder

# 2. Algorithm Flowchart

# 3. Overflow Condition

Overflow occurs in division when the quotient cannot be represented in the available number of bits. In restoring division, overflow is detected BEFORE beginning the algorithm.

## 3.1 Overflow Detection Rule

Overflow Condition:  A[initial] >= M  (i.e., if the upper half of the dividend >= divisor)

If the upper n bits of the 2n-bit dividend are greater than or equal to the divisor, the quotient would require more than n bits — causing overflow.

## 3.2 Divide-by-Zero Error

A special case of overflow: if the Divisor M = 0, division is undefined. The Verilog simulation output shows this as a 'Divide by zero error'.

From Verilog Output (Image 2):  'Divide by zero error'

## 3.3 Overflow Detection Table

| Dividend | Divisor | Upper Half | Condition | Status |
| --- | --- | --- | --- | --- |
| 14 (1110) | 3 (0011) | 0000 | 0000 < 0011 | OK |
| 9 (1001) | 2 (0010) | 0000 | 0000 < 0010 | OK |
| 7 (0111) | 3 (0011) | 0000 | 0000 < 0011 | OK |
| 78 (0100 1110) | 17 (0001 0001) | 0000 | 0000 < 0001 | OK |
| 15 (1111) | 1 (0001) | 0111 | 0111 > 0001 | OVERFLOW |
| Any | 0 (0000) | — | M = 0 | DIV/0 |

# 4. Worked Example 1 — 14 ÷ 3  (From Verilog Simulation)

This example matches exactly the Verilog output shown in Image 1.

## 4.1 Setup

| Parameter | Decimal | Binary (4-bit) | Notes |
| --- | --- | --- | --- |
| Dividend (Q) | 14 | 1110 | Loaded into Q reg |
| Divisor (M) | 3 | 0011 | Subtracted each step |
| Accumulator (A) | 0 | 00000 | 5-bit: MSB is sign |
| Expected Quotient | 4 | 0100 |  |
| Expected Remainder | 2 | 0010 |  |

## 4.2 Step-by-Step Trace

| Step | Operation | A (Accumulator) | Q (Quotient Reg) | Action |
| --- | --- | --- | --- | --- |
| Init | Load Q=1110, A=00000 | 00000 | 1110 | Initialize |
| Step 1 | Left Shift | 00001 | 1100 | Shift |
| Step 1 | A-M = 00001-00011 | 11110 (30) | 1100 | Subtract |
| Step 1 | MSB=1 → RESTORE A=A+M | 00001 | 1100 | Restore → Q[0]=0 |
| Step 1 End |  | 00001 | 1100 |  |
| Step 2 | Left Shift | 00011 | 1000 | Shift |
| Step 2 | A-M = 00011-00011 | 00000 (0) | 1000 | Subtract |
| Step 2 | MSB=0 → KEEP | 00000 | 1001 | Keep → Q[0]=1 |
| Step 2 End |  | 00000 | 1001 |  |
| Step 3 | Left Shift | 00001 | 0010 | Shift |
| Step 3 | A-M = 00001-00011 | 11110 (30) | 0010 | Subtract |
| Step 3 | MSB=1 → RESTORE | 00001 | 0010 | Restore → Q[0]=0 |
| Step 3 End |  | 00001 | 0010 |  |
| Step 4 | Left Shift | 00010 | 0100 | Shift |
| Step 4 | A-M = 00010-00011 | 11111 (31) | 0100 | Subtract |
| Step 4 | MSB=1 → RESTORE | 00010 | 0100 | Restore → Q[0]=0 |
| Step 4 End |  | 00010 | 0100 |  |
| Final Quotient Q = Q = 0100 = 4 |  |  | Remainder R = A = 0010 = 2 |  |

Verilog Output confirms: Final Q=0100 (4)  R=0010 (2)  ✓

# 5. Worked Example 2 — 9 ÷ 2  (From Verilog Simulation)

This example matches the Verilog output shown in Image 2.

## 5.1 Setup

| Parameter | Decimal | Binary (4-bit) | Notes |
| --- | --- | --- | --- |
| Dividend (Q) | 9 | 1001 | Loaded into Q reg |
| Divisor (M) | 2 | 0010 |  |
| Expected Quotient | 4 | 0100 |  |
| Expected Remainder | 1 | 0001 |  |

## 5.2 Step-by-Step Trace

| Step | Operation | A (Accumulator) | Q (Quotient Reg) | Action |
| --- | --- | --- | --- | --- |
| Init | Load Q=1001, A=00000 | 00000 | 1001 | Initialize |
| Step 1 | Left Shift | 00001 | 0010 | Shift |
| Step 1 | A-M = 00001-00010 | 11111 (31) | 0010 | Subtract |
| Step 1 | MSB=1 → RESTORE | 00001 | 0010 | Restore → Q[0]=0 |
| Step 1 End |  | 00001 | 0010 |  |
| Step 2 | Left Shift | 00010 | 0100 | Shift |
| Step 2 | A-M = 00010-00010 | 00000 (0) | 0100 | Subtract |
| Step 2 | MSB=0 → KEEP | 00000 | 0101 | Keep → Q[0]=1 |
| Step 2 End |  | 00000 | 0101 |  |
| Step 3 | Left Shift | 00000 | 1010 | Shift |
| Step 3 | A-M = 00000-00010 | 11110 (30) | 1010 | Subtract |
| Step 3 | MSB=1 → RESTORE | 00000 | 1010 | Restore → Q[0]=0 |
| Step 3 End |  | 00000 | 1010 |  |
| Step 4 | Left Shift | 00001 | 0100 | Shift |
| Step 4 | A-M = 00001-00010 | 11111 (31) | 0100 | Subtract |
| Step 4 | MSB=1 → RESTORE | 00001 | 0100 | Restore → Q[0]=0 |
| Step 4 End |  | 00001 | 0100 |  |
| Final Quotient Q = Q = 0100 = 4 |  |  | Remainder R = A = 0001 = 1 |  |

Verilog Output confirms: Final Q=0100 (4)  R=0001 (1)  ✓

# 6. OUTPUT:

7. Comparison Chart — All Worked Examples

| Dividend | Divisor | Binary Operation | Quotient | Remainder | Source |
| --- | --- | --- | --- | --- | --- |
| 14 | 3 | 1110 ÷ 0011 | 0100 = 4 | 0010 = 2 | Verilog Img 1 |
| 9 | 2 | 1001 ÷ 0010 | 0100 = 4 | 0001 = 1 | Verilog Img 2 |
| 7 | 3 | 0111 ÷ 0011 | 0010 = 2 | 0001 = 1 | Verilog Img 3 |
| 78 | 17 | 01001110 ÷ 00010001 | 00000100 = 4 | 00001010 = 10 | Manual (8-bit) |
| Any | 0 | — ÷ 00000000 | N/A | N/A | Div/Zero Error |

# 8. Restoring Division — Action Decision Chart

| Condition | MSB of A | Sign of A | Action | Q[0] Set |
| --- | --- | --- | --- | --- |
| A - M >= 0 | 0 | Positive | KEEP (no restore) | 1 |
| A - M < 0 | 1 | Negative | RESTORE: A=A+M | 0 |
| M = 0 | N/A | N/A | Divide by Zero ERROR | — |
| Overflow (A>=M initially) | N/A | N/A | OVERFLOW, halt | — |

# 10. Verilog Simulation — Key Code Snippet

module restoring_division;

parameter N = 4;

reg [N-1:0] dividend;

reg [N-1:0] divisor;

reg [N:0] A;

reg [N-1:0] Q;

reg [N-1:0] M;

reg [N-1:0] quotient;

reg [N-1:0] remainder;

reg error;

integer i;

task run;

begin

A = 0;

Q = dividend;

M = divisor;

error = 0;

if (M == 0) begin

error = 1;

quotient = 0;

remainder = 0;

$display("\nDivide by zero error\n");

end

else begin

$display("\n===== NEW DIVISION =====");

$display("Q=%b (%0d)  M=%b (%0d)", Q, Q, M, M);

for (i = 0; i < N; i = i + 1) begin

A = {A[N-1:0], Q[N-1]};

Q = {Q[N-2:0], 1'b0};

$display("Step %0d Shift   A=%b Q=%b", i+1, A, Q);

A = A - M;

$display("Step %0d A-M     A=%b (%0d)", i+1, A, A);

if (A[N] == 1) begin

A = A + M;

Q[0] = 0;

$display("Step %0d Restore", i+1);

end

else begin

Q[0] = 1;

$display("Step %0d Keep", i+1);

end

$display("Step %0d End     A=%b Q=%b\n", i+1, A, Q);

end

quotient = Q;

remainder = A[N-1:0];

if (quotient > dividend)

$display("Overflow detected");

$display("Final Q=%b (%0d)  R=%b (%0d)\n",

quotient, quotient, remainder, remainder);

end

end

endtask

initial begin

dividend = 4'b1110;

divisor  = 4'b0011;

run();

dividend = 4'b1001;

divisor  = 4'b0010;

run();

dividend = 4'b1010;

divisor  = 4'b0000;

run();

dividend = 4'b0111;

divisor  = 4'b0011;

run();

$finish;

end

endmodule

# 11. Summary

| Feature | Description |
| --- | --- |
| Algorithm Type | Sequential shift-and-subtract, n iterations for n-bit numbers |
| Hardware | Adder/Subtractor, two registers (A and Q), counter |
| Time Complexity | O(n) — one step per bit of dividend |
| Overflow Detection | Check upper half of A against M before starting |
| Divide-by-Zero | Check M != 0 before starting; flag error if M = 0 |
| Restoring Step | When subtraction gives negative result, add back divisor (restore A) |
| Final Result | Quotient in Q register; Remainder in A accumulator |

— End of Document —